|  | Design Final Roadway Design Plan Review |  |  |
| --- | --- | --- | --- |
| Instructions for Use: This form is to be completed by the LAD PC to check Final Plan Set submitted to NDOT by LPA. |  |  |  |
| Local Public Agency (LPA): |  | LPA Responsible Charge: |  |
| State Project No.: |  | Project Name and Location: |  |
| State Control No.: |  | Date of Review: | This Form was Completed By: |

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 1. | Has the designer incorporated the comments from the Public Hearing (if held) in the plans? |  |  |  |  | Public hearing transcript |  |
| 2. | Has the designer incorporated the comments from the review of the Functional Design plan submittal? |  |  |  |  | Functional Design Plans and 06-45 Review checklist |  |
| 3. | Have the required design relaxations been obtained? |  |  |  |  | Design relaxations |  |
| 4. | Do the plans reference the correct Project and Control Numbers? |  |  |  |  |  |  |
| 5. | Is there a north arrow shown on all applicable plan sheets? |  |  |  |  |  |  |
| 6. | Are the plan sheets assembled in the correct order? |  |  |  |  | PS&E Required Sheet |  |
| 7. | Does the title sheet list the design standard? |  |  |  |  |  |  |
| 8. | Does the title sheet list the correct traffic data? |  |  |  |  | Traffic Report |  |
| 9. | Does the title sheet have a location map? |  |  |  |  |  |  |
| 10. | Does the title sheet contain an index of all the sheets sufficient to locate all plan sheets? |  |  |  |  |  |  |
| 11. | Does plan set have the survey control point sheet ? |  |  |  |  |  |  |
| 12. | Does the location map correctly identify the begin and ending location for the project and for construction? |  |  |  |  |  |  |
| 13. | Do the typical sections represent all of the roadways to be constructed? |  |  |  |  |  |  |
| 14. | Do the stationing limits for shown typical section match the section of work being completed? |  |  |  |  |  |  |
| 15. | Do the typical sections accurately represent the pavement determination? |  |  |  |  | Pavement Design Determination |  |
| 16. | Do the typical sections include typicals for special details and transitions? |  |  |  |  |  |  |
| 17. | Does the legend on the typical section sheets match what’s shown in the graphic? |  |  |  |  |  |  |
| 18. | Are the dimensions on the typical section sheets for lane widths, side slopes, and obstacle clearances listed accurately? |  |  |  |  | Appropriate Design Standards (see LPA Guidelines Manual CH 6) |  |
| 19. | Is the detour shown on a detail sheet? |  |  |  |  |  |  |
| 20. | Are the wetland delineations clearly shown on the 2W sheets? |  |  |  |  | Wetland Delineation Report |  |
| 21. | Are the channels clearly shown on the 2W sheets? |  |  |  |  | Wetland Delineation Report |  |
| 22. | Are the limits of disturbance , do not disturb, limits of construction and restricted areas clearly shown on the 2W sheets? |  |  |  |  | Wetland Delineation Report Erosion Control Review Green Sheet |  |
| 23. | Do the plans contain Summary of Soils and Materials Sheets (2K sheets)? |  |  |  |  | 2K sheets are required for new construction and reconstruction projects |  |
| 24. | Do the phasing sheets provide for constructability and mobility? |  |  |  |  |  | This item review is to be performed by the LAD Section Head |
| 25. | On the 2L sheets, do the “Construction & Removal tabulars,” have corresponding symbols, stationing, offsets and quantities? |  |  |  |  |  |  |
| 26. | On the 2L sheets, are the limits of construction shown? |  |  |  |  |  |  |
| 27. | On the 2L sheets, are the drainage items shown (ie, structure size, length, location, position, type)? |  |  |  |  |  |  |
| 28. | On the 2L sheets, do the drainage items match the construction and removal tabulars? |  |  |  |  |  |  |
| 29. | On the Plan & Profile sheets, are the grades consistent with the minimum design standard? |  |  |  |  | Appropriate Design Standards (see LPA Guidelines Manual CH 6) |  |
| 30. | Do the plans show the information for horizontal curvature: station of PC, PI, PT, length of curve, deflection angle, degree of curve or radius, superelevation, length of tangent? |  |  |  |  |  |  |
| 31. | Do the design values for the horizontal curves meet the minimum design criteria? |  |  |  |  |  |  |
| 32. | Do the plans show the information for vertical curvature: VPI station, Length of Curve, K-value, VPI elevation? |  |  |  |  |  |  |
| 33. | Do the design values for the vertical curves meet the minimum design criteria? |  |  |  |  |  |  |
| 34. | On the plan and profile sheets is the topography legible? |  |  |  |  |  |  |
| 35. | Do the plan and profile sheets show the design for intersecting roadways and drives? |  |  |  |  |  |  |
| 36. | Do the plan and profile sheets depict the design of special ditches? |  |  |  |  |  |  |
| 37. | Do the plans show the ownership / property lines? |  |  |  |  | Required for projects where property rights are acquired |  |
| 38. | Do the plans depict the locations of known utilities? |  |  |  |  |  |  |
| 39. | If there are changes to Plans that affect Utilities, have the Utilities been notified, and re-inspection conducted? |  |  |  |  |  |  |
| 40. | Do the plans represent the locations where permanent accommodation is required? |  |  |  |  | PROWAG, ADAAG, Designing Sidewalks and trails for accessibility, DOT-OI 60-10 | This item to be checked by the LAD Section Head |
| 41. | Are items to be salvaged identified? |  |  |  |  |  |  |
| 42. | Are the existing culverts noted on the plans? |  |  |  |  |  |  |
| 43. | Do the culvert cross sections indicate the culvert invert on the inlet and outlet, the length of culvert, deflection of elbows, end treatments, number of barrels or pipes and pay quantities? |  |  |  |  |  |  |
| 44. | Do the culvert notes on the plan and profile sheets match the information on the culvert cross section sheets? |  |  |  |  |  |  |
| 45. | Do the culvert designs for bridge size structures include the hydraulic design information in the notes? |  |  |  |  |  |  |
| 46. | Do the plans provide for drainage in the low areas such as sag vertical curves, the low side of super elevated curves with raised median? |  |  |  |  |  | This item review is to be performed by the LAD Section Head |
| 47. | Do the plans provide adequate drainage for the travelled way on high speed roadways? |  |  |  |  |  | This item review is to be performed by the LAD Section Head |
| 48. | Are all of the hazards in the fixed obstacle clearance or clear zone removed, shielded or shown as exceptions to the design standards? |  |  |  |  |  |  |
| 49. | Does the contractor have enough room to do the required earthwork/construction tasks? |  |  |  |  |  |  |
| 50. | Are all of the guardrail locations described in the build notes and shown on the plans? |  |  |  |  |  |  |
| 51. | Is there documentation that provided by the designer that indicates that barriers, bridge curbs, breakaway signs within the clear zone are acceptable for use as per NCHRP 350 or MASH-09. |  |  |  |  |  |  |
| 52. | Is erosion control shown in the plans? |  |  |  |  |  |  |
| 53. | Do the plans show seeding? |  |  |  |  |  |  |
| 54. | Are any trees to be placed, planned to be placed sufficiently outside the lateral clear zone such that a mature tree canopy and trunk will not encroach in the clear zone? |  |  |  |  |  |  |
| 55. | Do the plans show the locations of Railroads? |  |  |  |  |  |  |
| 56. | Are 4(f) lands, 6(f) lands locations and impacts identified? |  |  |  |  | Green Sheet |  |
| 57. | Are locations of historical property and impacts identified? |  |  |  |  | Green Sheet |  |
| 58. | Does the plan set reference the current version of the standard plans? |  |  |  |  |  |  |
| 59. | Do the plan and profile sheets represent accurately the TS&L of the structures? |  |  |  |  | Bridge Plans |  |
| 60. | If parking is included in the project, does the parking meet the requirements of the ADA for number of ADA stalls, size and placement of the stalls? |  |  |  |  |  |  |
| 61. | Do the earthwork cross sections show: station, elevations of existing and new ground, special ditches and right of way limit? |  |  |  |  |  |  |
| 62. | Do the earthwork cross sections match the typical sections? |  |  |  |  |  |  |
| 63. | Are limits of construction within the right way as shown on the right of way sheets? |  |  |  |  |  |  |
| 64. | Do the plans show the permanent signing and pavement marking? |  |  |  |  |  |  |
| 65. | Does the permanent signing and marking comply with the MUTCD and Nebraska Supplement to the MUTCD? |  |  |  |  |  |  |
| 66. | Do the plans contain all of the necessary plans sheet to sufficiently describe the work and the construction details of the project? |  |  |  |  |  |  |