|  | Design Functional Design Plan Review Checklist |  |  |
| --- | --- | --- | --- |
| Instructions for Use: This form is to be completed by LAD PC to review functional plans, which are prepared after the plan in hand meeting. |  |  |  |
| Local Public Agency (LPA): |  | LPA Responsible Charge: |  |
| State Project No.: |  | Project Name and Location: |  |
| State Control No.: |  | Date of Review: | This Form was Completed By: |

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 1. | Has the designer incorporated the comments from the Public Information meeting (if held) in the plans? |  |  |  |  | Public Meeting Minutes |  |
| 2. | Has the designer incorporated the comments from the review of the plan in hand (30%) plan submittal? |  |  |  |  | Plan in Hand (30%) Plans |  |
| 3. | Do the plans reference the correct Project and Control Numbers? |  |  |  |  |  |  |
| 4. | Do the plans contain title, typical section, typical details, plan and profile, wetland, culvert cross section and earthwork cross section sheets? |  |  |  |  |  |  |
| 5. | Is there a north arrow shown on all applicable plan sheets? |  |  |  |  |  |  |
| 6. | Are the plan sheets assembled in the correct order? |  |  |  |  | Chap 6 LPA Manual |  |
| 7. | Does the title sheet list the design standard? |  |  |  |  | Minimum Design Standards |  |
| 8. | Does the title sheet list the design traffic data? |  |  |  |  | Information provided by NDOT Traffic Data Memo |  |
| 9. | Does the title sheet list the design speed? |  |  |  |  | Design Criteria Sheet |  |
| 10. | Does the title sheet have a location map? |  |  |  |  |  |  |
| 11. | If the project will require a detour, is the detour shown on detail sheet? |  |  |  |  |  |  |
| 12. | Does the typical section accurately represent the lane widths, depth of surfacing side slopes, sidewalks and obstacle clearances, and shoulder width? |  |  |  |  | Pavement determination |  |
| 13. | Do the typical sections represent all of the roadway sections to be constructed? |  |  |  |  |  |  |
| 14. | Do the typical sections include special details and transitions? |  |  |  |  |  |  |
| 15. | Do the legend items on the typical sections match what’s shown in the typical graphic? |  |  |  |  |  |  |
| 16. | Do the project limits match the section of work being completed? |  |  |  |  | NDOT 530 |  |
| 17. | On the Plan & Profile sheets, are the grades consistent with the minimum design standard? |  |  |  |  | Design Criteria Sheet |  |
| 18. | Do the plans depict the appropriate information for the horizontal curvature; station of PC, PI, PT, length, super elevation, tangent length? |  |  |  |  |  |  |
| 19. | On the Plan & Profile sheets, do the design values of the horizontal curves meet the minimum design criteria? |  |  |  |  | Design Criteria Sheet |  |
| 20. | Do the plans depict the appropriate information for the vertical curvature, VPI station, Length of Curve, K-value, VPI elevation? |  |  |  |  |  |  |
| 21. | On the Plan & Profile sheets, do design values of the vertical curves meet the minimum design criteria? |  |  |  |  | Design Criteria Sheet |  |
| 22. | On the plan and profile sheets is the existing topography shown and legible? |  |  |  |  |  |  |
| 23. | Have all of the intersecting roadways and drives been designed and shown on the plans? |  |  |  |  |  |  |
| 24. | Are the proper radii used for the returns of intersecting roadways? |  |  |  |  |  |  |
| 25. | Do the plans show the property lines and ownership details? |  |  |  |  |  |  |
| 26. | Do the plans show the locations of all known utilities? |  |  |  |  |  |  |
| 27. | If there are changes to Plans that affect Utilities, have the Utilities been notified, and reinspection conducted? |  |  |  |  |  |  |
| 28. | Do the plan and profile sheets accurately identify channels? |  |  |  |  | Survey and/or Wetlands Report |  |
| 29. | On the 2L sheets, do the “Construction & Removal” tabulars have accurate symbols, stationing, offsets and quantities? |  |  |  |  |  |  |
| 30. | On the 2L sheets, are the limits of construction clearly shown? |  |  |  |  |  |  |
| 31. | On the 2L sheets, are the drainage items defined accurately such as noting the correct: elevations, slopes, flow lines, stationing, symbology, numbering and pipe size? |  |  |  |  |  |  |
| 32. | On the 2L sheets, do the drainage items match the construction and removal tabulars? |  |  |  |  |  |  |
| 33. | Do the plans represent the locations where permanent accommodation is required? |  |  |  |  | PROWAG, ADAAG, Designing Sidewalks and trails for accessibility, DOT OI 60-10 | This item to be checked by the LAD Section Head |
| 34. | Do the culvert notes have the appropriate hydraulic information? |  |  |  |  |  |  |
| 35. | Are items to be salvaged clearly identified? |  |  |  |  |  |  |
| 36. | Are the existing culverts noted on the plans (station, size and type)? |  |  |  |  | Survey |  |
| 37. | Are the new culverts shown on the plans(station, size and type)? |  |  |  |  |  |  |
| 38. | Do the culvert cross sections clearly indicate the culvert invert on the inlet and outlet, the length of culvert, deflection of elbows, end treatments, and excavation quantities? |  |  |  |  |  |  |
| 39. | Do the culvert notes on the plan and profile sheets match the information on the culvert cross section sheets? |  |  |  |  |  |  |
| 40. | Do the culvert designs for bridge size structures include the hydraulic design information in the notes? |  |  |  |  |  |  |
| 41. | Are all of the hazards in the fixed obstacle clearance or clear zone removed, shielded or shown as exceptions to the design standards? |  |  |  |  | Minimum Design Standards and Exceptions |  |
| 42. | Are all of the guardrail locations properly depicted and described in the build notes and on the plans? |  |  |  |  |  |  |
| 43. | If barriers are used in the permanent design, are they crashworthy? |  |  |  |  | MASH 09 |  |
| 44. | Are the erosion control plans shown, and do they use appropriate symbols? |  |  |  |  |  |  |
| 45. | Do the plans depict the locations of Railroads? |  |  |  |  |  |  |
| 46. | Are wetlands locations and preliminary impacts adequately identified? |  |  |  |  |  |  |
| 47. | Are 4(f) and 6(f) lands located, and impacts adequately identified? |  |  |  |  |  |  |
| 48. | Are historical properties’ locations and impacts adequately identified? |  |  |  |  |  |  |
| 49. | Do the earthwork calculations represent widening necessary for guardrail, culverts, driveways and other miscellaneous excavations or embankments? |  |  |  |  |  |  |
| 50. | Does the contractor have enough room to do the required earthwork/construction tasks? |  |  |  |  | Check with PS&E if questionable |  |
| 51. | Do the plan and profile sheets represent accurately the TS&L of the structures? |  |  |  |  | Bridge Data Sheets |  |
| 52. | If parking is included in the project, does the parking meet the requirements of the ADA for number of ADA stalls, size and placement of the stalls? |  |  |  |  |  |  |
| 53. | Do the earthwork cross sections contain station, elevations of existing and new ground, and special ditches? |  |  |  |  |  |  |
| 54. | Are the earthwork cross sections consistent with the typical sections? |  |  |  |  |  |  |
| 55. | Does the estimate include all identified items and reasonable quantities? |  |  |  |  |  |  |
| 56. | For those items not specifically identified, are there reasonable lump sum estimates of the costs? |  |  |  |  | Check with Estimating Section if questionable |  |
| 57. | Is the basis for the unit price identified and is it reasonable? |  |  |  |  | NDOT AUP List |  |