|  | 90% LPA Bridge Plan Review Checklist LAD PC is responsible for completing boxes shaded in gray prior to routing of checklist. |  |  |
| --- | --- | --- | --- |
| Local Public Agency (LPA): |  | LPA Responsible Charge: | PE Consultant: |
| State Project No.: |  | Project Name and Location: |  |
| State Control No.: |  | Date Received by NDOT: | Structure No: |
| This form was completed by (insert name and date review was completed): |  |  |  |
| NDOT Local Assistance Division Project Coordinator : |  | Date: |  |
| NDOT Materials and Research Geotechnical Section: |  | Date: |  |
| NDOT Bridge Design Section: |  | Date: |  |

Instructions for Use: For the purpose of this review, “90% Bridge Plans” shall be understood to mean Bridge Plans that have been both designed and independently checked. All associated detailing, including complete pay item listing, shall be completed and shown on the plans. Quantity Calculations and Load Rating will NOT be reviewed at this time.

This section will be completed by the NDOT Materials and Research Geotechnical Section.

Pile Layout

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 1. | Are the interior and overall pile dimensions shown? (pile spacing) |  |  |  |  | 70% Plans |  |
| 2. | Are the substructure piling numbered and indicate batter, if required? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/ Plans |  |

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 3. | Are the existing, undimensioned piling shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |

Pile Specifications

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 4. | Are the splice details for H-Pile and pipe pile shown, if required? |  |  |  |  | BOPP Manual/Plans |  |
| 5. | If required, is the concrete pile detail shown? |  |  |  |  | BOPP Manual |  |
| 6. | Is the alternate pile table for concrete pile shown, if required? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 7. | Is the test pile table shown, if required? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 8. | Are the general pile notes from BOPP manual shown? |  |  |  |  | BOPP Manual/Plans |  |

Elevation and Plan View Sheet Information

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 9. | Are the pile drawn to scale? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 10. | Is the pile capacity the same as recommended (or less than)? |  |  |  |  | Bridge Foundation Design File/Plans |  |

Bridges with MSE Walls*

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 11. | Is the CMP sleeve table and appropriate notes shown? |  |  |  |  | BOPP Manual/Plans |  |
| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
|  |  | Yes | No | N/A |  |  |  |
| 12. | Is the delay period note shown, if needed? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 13. | Is the MSE note shown? (Abutment and grade beam piling may be.) |  |  |  |  |  |  |

Bridge Widening

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 14. | Is the existing pile data table shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |

Drilled Shafts/Rock Sockets

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 15. | Is the rebar correctly sized and spaced? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 16. | Is the bottom of socket or shaft elevation shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 17. | Is the bottom and side clear cover correct? |  |  |  |  | BOPP Manual/Bridge Foundation Design File |  |
| 18. | Is the temporary or permanent casing shown and of proper length and thickness? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 19. | Are the limits of rock socket shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 20. | Are the limits of the shaft shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 21. | Are the cross sections shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 22. | Is the top of rock elevation shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |

Spread Footings

| Item # | Task Description or Questions | Completed |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |  |  |
| 23. | Is the factored bearing capacity shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 24. | Are the footing details shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 25. | Is the rebar configuration shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |
| 26. | Are the appropriate notes shown? |  |  |  |  | BOPP Manual/Bridge Foundation Design File/Plans |  |

This Section will be completed by the NDOT Bridge Division Design Section.

| Item # | Task Description or Questions |  | Completed |  |  |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Yes |  | No |  | N/A |  |  |  |
| 27. | Has the information on the Data Sheet/TS & L been properly incorporated into the 90% plans? |  |  |  |  |  |  |  | Final TS&L Sheets Bridge Data Sheets Bridge Plans |  |
| 28. | Review channel and riprap details. If different from the Data Sheet/TS & L, has the Hydraulic Section reviewed and accepted the changes? |  |  |  |  |  |  |  | Final TS&L Sheet Bridge Data Sheet Bridge Plans |  |
| 29. | Calculate the freeboard available and then compare it to the freeboard required. Is it acceptable? |  |  |  |  |  |  |  | Final TS&L Sheet Bridge Data Sheets Bridge Plans |  |
| 30. | Review the horizontal and vertical clearances. Are they acceptable? |  |  |  |  |  |  |  | NDOT Minimum Design Std Bridge Plans BOPP |  |
| Item # | Task Description or Questions |  | Completed |  |  |  |  | If No, Define Corrective Action | Details or Information Used to Verify Content | Additional Comments |
|  |  |  | Yes |  | No |  | N/A |  |  |  |
| 31. | Perform a general plan review for adherence to the project scope. Is everything acceptable? |  |  |  |  |  |  |  | BOPP Project Scope LAD Project Coordinator Bridge Plans |  |
| 32. | Review the Special Provisions. Are they acceptable? Note: Include references to lead paint as applicable. |  |  |  |  |  |  |  | BOPP Project Scope Bridge Plans |  |
| 33. | Review the pay items specified. Are they acceptable? |  |  |  |  |  |  |  | BOPP Bridge Plans |  |
| Comments: |  |  |  |  |  |  |  |  |  |  |